### ANNEXURE II

PART A : TECHNICAL BID

- Consolidated Bill of Material of Equipments/Services Offered.

| S.No. | TECHNICAL BID : Schedule of Material quoted, without Costs. |  |  | Unit Supply Cost (Rs.) |  | QTY |  | UoM | Total Unit SUPPLY Cost incl.Taxes (Rs.)(A) | Goods & Service Tax (%) | Goods & Service Tax (%) (Rs.) | TOTAL SUPPLY Cost incl.Taxes (Rs.)(A x QTY) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Detailed Bill of Materails w/o COST to be provided as part of TECHNICAL BID by BIDDER) |  |  |  |  |  |  |  |  |  |  |  |
| A | ACTIVE Components |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Dual-Processor Rack-Mount Servers |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 2 | RHEL Server Operating System SW for VMs |  |  |  |  | 16 |  | nos. |  |  |  |  |
| 3 | RHEL Workstation Operating System SW |  |  |  |  | 2 |  | nos. |  |  |  |  |
| 4 | WINDOWS Server Operating System SW for VMs |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 5 | WINDOWS Server Operating System SW |  |  |  |  | 2 |  | nos. |  |  |  |  |
| 6 | VMWare VSphere SW |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 7 | Server Rack, 42U, 600mm x 1200mm |  |  |  |  | 4 |  | nos. |  |  |  |  |
|  | Documents, including testing and commissioning certificates, and warranty certificates. |  |  |  |  |  |  |  |  |  |  |  |
|  | TOTAL (in figures) |  |  |  |  |  |  |  |  |  |  |  |
|  | TOTAL (in words) |  |  |  |  |  |  |  |  |  |  |  |
|  | Bidder’s Office Seal |  |  |  |  |  | Signature with Date |  |  |  |  |  |
|  | Name of Bidder |  |  |  |  |  | Name of Authorised Signatory of Bidder: |  |  |  |  |  |

PART A : TECHNICAL  BID

- Detailed Bill of Material of Equipments/Services Offered.

| S.No. |  | Equipments/Services Required | Part |  |  |  |  | Mfr/Model Name | Mfr/Model Number | Qty offered. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Dual-Processor Rack-Mount Servers | Chassis |  |  |  |  |  |  |  |
|  |  |  | Processor |  |  |  |  |  |  |  |
|  |  |  | Motherboard |  |  |  |  |  |  |  |
|  |  |  | Memory |  |  |  |  |  |  |  |
|  |  |  | Disk Drives |  |  |  |  |  |  |  |
|  |  |  | Optical Drives |  |  |  |  |  |  |  |
|  |  |  | RAID Controller |  |  |  |  |  |  |  |
|  |  |  | Network Connectivity |  |  |  |  |  |  |  |
|  |  |  | External Storage Connectivity |  |  |  |  |  |  |  |
|  |  |  | Data Connectivity |  |  |  |  |  |  |  |
|  |  |  | Power Supply |  |  |  |  |  |  |  |
|  |  |  | Cooling Fans |  |  |  |  |  |  |  |
|  |  |  | System Security |  |  |  |  |  |  |  |
|  |  |  | Embedded Remote Management |  |  |  |  |  |  |  |
|  |  |  | Server Management |  |  |  |  |  |  |  |
|  |  |  | OEM Assurance |  |  |  |  |  |  |  |
|  |  |  | OEM Warranty |  |  |  |  |  |  |  |
| 2 |  | RHEL Server Operating System SW for VMs |  |  |  |  |  |  |  |  |
| 3 |  | RHEL Workstation Operating System SW |  |  |  |  |  |  |  |  |
| 4 |  | WINDOWS Server Operating System SW for VMs |  |  |  |  |  |  |  |  |
| 5 |  | WINDOWS Server Operating System SW |  |  |  |  |  |  |  |  |
| 6 |  | VMWare VSphere Software |  |  |  |  |  |  |  |  |
| 7 |  | Server Rack, 42U, 600mm x 1200mm |  |  |  |  |  |  |  |  |
|  | Bidder’s Office Seal |  |  |  |  |  | Signature with Date |  |  |  |
|  | Name of Bidder |  |  |  |  |  | Name of Authorised Signatory of Bidder: |  |  |  |

PART A  : TECHNICAL  BID

- Compliance / Deviation of Technical Specifications of Equipments/Services Offered.

| S.No. | Equipments/Services Required | Part | Technical Specifications of Equipments/Services Required |  |  |  |  | Mfr/Model Name | Mfr/Model /Part Number | Qty offered | Deviation, if any. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dual-Processor Rack-Mount Servers | Chassis | 2U 19" Rack Mountable, and supplied with necessary OEM Sliding Rack-Mount Kit and Cable-Management Arm. |  |  |  |  |  |  |  |  |
|  |  | Processor | 2nos of Intel Xeon 2650v4 or later generation processors, each having at least 12-cores and processor base frequency of at least 2.2 GHz. |  |  |  |  |  |  |  |  |
|  |  | Motherboard | Intel® Series Chipset, with not more than 2 sockets. |  |  |  |  |  |  |  |  |
|  |  | Memory | 256GB DRAM, scalable to atleast 768GB using DDR4 operating at 2400 MHz or higher. |  |  |  |  |  |  |  |  |
|  |  | Disk Drive Bays | Atleast 16 hotplug drive bays, to install a combination of SFF or LFF HDDs, including atleast 4 SSDs. |  |  |  |  |  |  |  |  |
|  |  | Disk Drives | Shall support Hot-Plug SFF or LFF SATA/SAS/NL-SAS/SSD disk drives and NVMe drives. |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 3 nos of high-endurance SFF SSD, totaling atleast 600GB. |  |  |  |  |  |  |  |  |
|  |  |  | At least 3nos of 10K RPM SAS HDD totaling atleast 1.5 TB. |  |  |  |  |  |  |  |  |
|  |  |  | At least 3nos of 7.2K RPM NL-SAS/SATA HDD totaling atleast 6 TB. |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 7 free slots to add HDDs/SSDs for future addition. |  |  |  |  |  |  |  |  |
|  |  | Optical Drives | DVD Read/Write drive. |  |  |  |  |  |  |  |  |
|  |  | RAID Controller | RAID controller shall support multiple channels of SSDs and HDDs RAIDs. |  |  |  |  |  |  |  |  |
|  |  |  | PCIe 3.0 based 12Gb/s SAS Raid Controller with RAID 0/1/01/5/10 with 4GB battery-backed write cache. |  |  |  |  |  |  |  |  |
|  |  |  | RAID controller shall support RAID-passthrough/JBODs and large queue-depths to support VMWare VSAN hosting requirements. |  |  |  |  |  |  |  |  |
|  |  | Network Connectivity | 1Gbps and 10Gbps Ethernet connectivity through RJ45 network adaptors and Cat6 cables. |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 4nos of 1Gbps full-duplex interfaces. |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 2nos of 10Gbps full-duplex dual-port interfaces. |  |  |  |  |  |  |  |  |
|  |  |  | Include suitable network cables to interconnect with 10GBase-T RJ45/SFP ports of 10Gbps ethernet Switch. |  |  |  |  |  |  |  |  |
|  |  |  | Shall support iSCSI, FCoE, and Virtualisation. |  |  |  |  |  |  |  |  |
|  |  | External Storage Connectivity | Support Fibre Channel Host Bus Adaptor, with multipaths for no single point failure. |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 2nos of dual-port 16Gbps auto-negotiable Fibre Channel HBA. |  |  |  |  |  |  |  |  |
|  |  |  | Include suitable fibre cables to interconnect with 8Gbps FC ports of SAN Switch. |  |  |  |  |  |  |  |  |
|  |  |  | Interface shall be compatible with existing SAN/SAN Switch. |  |  |  |  |  |  |  |  |
|  |  |  | Interfaces shall support Virtualisation. |  |  |  |  |  |  |  |  |
|  |  | Data Connectivity | Serial port. |  |  |  |  |  |  |  |  |
|  |  |  | Micro SD slot (desirable). |  |  |  |  |  |  |  |  |
|  |  |  | Atleast 5nos of USB 3.0 interfaces, of which atleast 2 are external interfaces. |  |  |  |  |  |  |  |  |
|  |  | PCI Bus Slots | At least six PCI-Express 3.0 slots Atleast two x16 PCIe slots. |  |  |  |  |  |  |  |  |
|  |  | Video Port | Shall have 1 VGA/HDMI video port with cache. |  |  |  |  |  |  |  |  |
|  |  | Power Supply | Hot-plug high-efficiency redundant power supplies that shall power fully-loaded configuration of server, including hard drives. Shall be rated for Indian standards. |  |  |  |  |  |  |  |  |
|  |  | Cooling Fans | Redundant hot-plug system fans to cool fully-loaded configuration of server, including hard drives. |  |  |  |  |  |  |  |  |
|  |  | Industry Standard Compliance | ACPI 6.1, PCIe 3.0 compliant. Support PXE, WOL, USB 3.0/2.0, Energy Star, ASHRAE A3/A4 |  |  |  |  |  |  |  |  |
|  |  | Operating Systems and Virtualization Software Support | Microsoft Windows Server 2012 or later Red Hat Enterprise Linux (RHEL 6.0 or higher) Vmware Vsphere 6.5 and VSAN 6.0 or higher All server components to be compliant for virtualization under HCL of VMWare VSphere/VSAN. |  |  |  |  |  |  |  |  |
|  |  | System Security | Secure, authentic and verifiable upgrade of firware and software. Detect arbitrary or compromised firmware codes or algorithms. Enable roll-back of upgraded firmware. Chassis Intrusion Detection |  |  |  |  |  |  |  |  |
|  |  | Embedded Remote Management | Exclusive System Remote Management port to access embedded Web-based GUI console having features for virtual power-off/on, remote-boot etc. Remote Management port shall support Ethernet PIV4/IPV6/802.1Q VLAN. Update/upgrade of software, firmware and software patches from remote clients. Support agentless management using out-of-band remote management port. Support monitoring and recording changes in the server hardware and system configuration. Support management of multiple servers. Support embedded remote support to transmit hardware events to OEM. |  |  |  |  |  |  |  |  |
|  |  | Server Management | OEM Server Management Software to monitor and view consolidated inventory and status of server resources and monitor its health. Update Software, Firware of Server System and its components. Alert about deteriorating performance, developing faults and possible failures of all vital elements of the server like disk-drives, processor, memory etc. Push traps and syslog of events to external Management Servers. Integrate with VMWare Vcentre. Remote Media Access with Virtual Media Remote video, keyboard, and mouse control with Virtual Console |  |  |  |  |  |  |  |  |
|  |  | OEM Assurance | Equipment supplied shall be in the current manufacturing range of the OEM and have the latest technical features and specifications. OEM shall assure that the model of equipment quoted shall not be declared end-of-sale by the OEM for at least 2 years from date of supply. OEM shall assure that the model of equipment quoted shall not be declared end-of-support by the OEM for at least 7 years from date of supply. |  |  |  |  |  |  |  |  |
|  |  | OEM Warranty | OEM or the bidder shall provide Warranty for cost-free parts/labor repair/replacement of hardware and update of software of the equipment supplied on NBD basis for a period of 5 years from date of installation. OEM shall make available a public Internet URL that can be accessed 24x7 during warranty period for product/technical support information, and track status of warranty, support contract etc. |  |  |  |  |  |  |  |  |
| 2 | RHEL Server Operating System SW for VMs |  | Perpetual license/subscription of RedHat Enterpise Linux 6.0 or later version, with 3year updates, self/basic support, for operation of atleast 16 VMs in a cluster of virtualized hosts, each node having 2P/24C. |  |  |  |  |  |  |  |  |
| 3 | RHEL Workstation Operating System SW |  | Perpetual license/subscription of RedHat Enterprise Linux for WorkStation 6.0 or later version, with 3year updates, self/basic support, for operation in 1P/4C workstations. |  |  |  |  |  |  |  |  |
| 4 | WINDOWS Server Operating System SW for VMs |  | Perpetual license/subscriptions of Microsoft Windows Server 2012/16, Standard / Essential Edition, with 3year updates, self/basic support, for operation of 4 VMs in a cluster of virtualized hosts, each node having 2P/12C. |  |  |  |  |  |  |  |  |
| 5 | WINDOWS Server Operating System SW |  | Perpetual licenses of Microsoft Windows Server 2012/16, Standard / Essential Edition, with 3year updates, self/basic support, for operation in 2P/4C hosts. |  |  |  |  |  |  |  |  |
| 6 | VMWare VSphere Software |  | VMware VSphere ESX 6.5 Standard latest version license for 4N/8P, with 3year updates, self/basic support, compliant and interoperable with VCenter 6.5 and VSAN 6.6 or later. |  |  |  |  |  |  |  |  |
| 7 | Server Rack, 42U, 800mm x 1000mm |  | Server Rack, 19", 42U, 600x1200mm Floor |  |  |  |  |  |  |  |  |
|  |  |  | Adjustable Fully Recessible 19" Vertical Posts/Mounting Angles |  |  |  |  |  |  |  |  |
|  |  |  | Split pair of 19" Mounting Angles at Rear to accommodate equipment of different depths. |  |  |  |  |  |  |  |  |
|  |  |  | Hinged, Fully Perforated Steel Doors in Front and Rear. Vertical half-width Split Doors preferred. |  |  |  |  |  |  |  |  |
|  |  |  | Removable, non-perforated Steel Side Panels. |  |  |  |  |  |  |  |  |
|  |  |  | Steel Top Cover, with provision for Cable Entry |  |  |  |  |  |  |  |  |
|  |  |  | Steel Bottom Cover, with provision for Cable Entry |  |  |  |  |  |  |  |  |
|  |  |  | Top-Cover-mounted Ventilating Fans |  |  |  |  |  |  |  |  |
|  |  |  | Heavy Duty Castor Wheels with Brakes |  |  |  |  |  |  |  |  |
|  |  |  | 42U Cable Channel with Guides/Loops |  |  |  |  |  |  |  |  |
|  |  |  | Load-bearing capacity of atleast 800kgs. |  |  |  |  |  |  |  |  |
|  |  |  | Manufactured in India by APW, APC, Rittal, HP or Netrack |  |  |  |  |  |  |  |  |
|  |  |  | 2x 24-port PDU, with 15A/5A IEC Sockets, Power Indicator, MCB, 3m 32A Power Cable. |  |  |  |  |  |  |  |  |
|  | Bidder’s Office Seal |  |  |  |  |  | Signature with Date |  |  |  |  |
|  | Name of Bidder |  |  |  |  |  | Name of Authorised Signatory of Bidder: |  |  |  |  |

PART B : COMMERCIAL BID

Costs as per Schedule of Equipments/Services Required.

| S.No. | COMMERCIAL BID : Costs as per Schedule of Material. |  |  | Unit Supply Cost (Rs.) |  | QTY |  | UoM | Total Unit SUPPLY Cost incl.Taxes (Rs.)(A) | Goods & Service Tax (%) | Goods & Service Tax (%) (Rs.) | TOTAL SUPPLY Cost incl.Taxes (Rs.)(A x QTY) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Detailed Bill of Materails w/o COST to be provided as part of TECHNICAL BID by BIDDER) |  |  |  |  |  |  |  |  |  |  |  |
| A | ACTIVE Components |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Dual-Processor Rack-Mount Servers |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 2 | RHEL Server Operating System SW for VMs |  |  |  |  | 16 |  | nos. |  |  |  |  |
| 3 | RHEL Workstation Operating System SW |  |  |  |  | 2 |  | nos. |  |  |  |  |
| 4 | WINDOWS Server Operating System SW for VMs |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 5 | WINDOWS Server Operating System SW |  |  |  |  | 2 |  | nos. |  |  |  |  |
| 6 | VMWare VSphere SW |  |  |  |  | 4 |  | nos. |  |  |  |  |
| 7 | Server Rack, 42U, 600mm x 1200mm |  |  |  |  | 4 |  | nos. |  |  |  |  |
|  | Documents, including testing and commissioning certificates, and warranty certificates. |  |  |  |  |  |  |  |  |  |  |  |
|  | TOTAL (in figures) |  |  |  |  |  |  |  |  |  |  |  |
|  | TOTAL (in words) |  |  |  |  |  |  |  |  |  |  |  |
|  | Bidder’s Office Seal |  |  |  |  |  | Signature with Date |  |  |  |  |  |
|  | Name of Bidder |  |  |  |  |  | Name of Authorised Signatory of Bidder: |  |  |  |  |  |